Installer Requirements Policy

The Fulton County Health Department requires a soil and site evaluation conducted by a certified soil scientist for all new and repair septic systems.  The soil report must include a minimum of three borings (described in full) with a minimum of two borings in the proposed leach field.  A list of registered soil scientists can be obtained at the Fulton County Health Department or online at the http://oisc.purdue.edu/irss/roster.html. The soil scientist’s report must either be hand delivered, mailed, e-mailed, or faxed to the Health Department by the soil scientist.   The Fulton County Health Department will not accept soil reports from anyone other than the soil scientist.

The Fulton County Environmentalist will determine the requirements regarding the proposed septic system after reviewing the soil scientist’s report and regulations set forth in Ordinance 120108 and Rule 410 IAC 6-8.3.  The Fulton County Health Department will then issue a preliminary septic approval letter to the property owner.

Prior to the issuance of a permit the following items must be completed:

- The Installer must be certified to install a septic system in Fulton County per the rules and regulations set forth in Fulton County Ordinance 120108.

- The Installer must submit a septic system design plan to the Fulton County Health Department.

- The Installer must physically layout (stakeout) all components of the septic system design where the system is to be installed.

All septic installers are required to submit septic system design plans prior to the issuance of a septic permit.  The septic system design plans must contain the following items:

- Septic installer’s name, address and phone number

- Site address and property owner’s name

- Boring locations

- All parts of the system and a cross section of the system

- Property boundaries and dimensions

- Footprint of all existing or proposed structures-including driveways

- Slope

- Septic Tank capacity and manufacturer

- Dose Tank

  - Capacity, manufacturer and tank material

  - Effluent Pump manufacturer, model number and pump curve

  - Cross Section of the dose tank, depicting float settings

  - The pump and pedestal submersion height in inches

  - Total dose height in inches – the capacity from the off float to the on float

  - Freeboard

1. Alarm height in inches – the capacity from the on float to the alarm float

2. Reserve capacity height in inches – the capacity above the high water alarm to the invert of the inlet

- Residential Sewer pipe ASTM number, diameter, and SDR

- Effluent Filter manufacturer and model number

- Gravity Effluent pipe ASTM number, diameter, and SDR

- Gravity Header pipe ASTM number, diameter, and SDR

- Dose Delivery/Effluent Force Main

  - ASTM number, diameter, and SDR

  - Total length and the length that drains back

  - Pump discharge rate

  - Total volume calculation

  - Total dynamic head calculation

- Required Elevations (laser or transit)

  - Elevations at the beginning, middle and end of each run/lateral of the absorption field and non-traditional systems, including drainage (outlet and four corners)

  - Elevations of the septic tank (dose tank - where applicable)

  - Elevations of the distribution box

16. Compass directions (at least north)

17.	Wells

- Separation distance to closest component of the septic system

- Neighbor’s well separation distance to closest component of the septic system

18.	Gravity absorption system

- Number of trenches

- Separation distance

- If a stone system: the type of pipe, amount of pipe, size of the pipe being used, type of stone, amount of stone, and stone distributor.  In addition, the amount of stone below and on top of the pipe must be specified.  As well as the hole orientation of the pipe in the trench(es).  The holes in the laterals must be placed at 12 o’clock, 4 o’clock and 8 o’clock if a three hole lateral pipe is used or 4 o’clock and 8 o’clock if a two hole lateral pipe is used.  Furthermore, each lateral end must be capped.  (The hole within the laterals and capping of the ends of each lateral are required to obtain equal distribution throughout the entire length of the lateral(s) per Rule 410 IAC 6-8.3.)

d. 	Trench width

e.	Trench depth

f.	Trench length

g.   Perimeter tile depth (where applicable)

h. 	If chambers: manufacturer and model number

19. Dispersal Area

The Installer must physically stake out all the components of the proposed septic system.

Once all the components of the septic system are staked out and the septic system plans have been submitted, the Environmental Specialist will conduct an onsite evaluation of the property to verify that the layout and plans submitted meet the requirements set forth for the septic system design.

Once the onsite evaluation of the property has been conducted and the plans submitted approved the Installer will be notified regarding the issuance of the permit. If the onsite evaluation and/or the plans submitted do not meet the requirements set forth the Property Owner will be notified in writing regarding the reason(s) why the permit has been denied.

The Fulton County Health Department will NOT accept any sketch of the septic system prior to the Fulton County Environmentalist’s issuance of the preliminary septic approval letter.  Any drawing submitted prior to the issuance of the preliminary septic approval letter will be returned to the Installer.

The Fulton County Health Department requires all installers to follow ALL the rules and requirements set forth in the attached Indiana State Department of Health’s list of acceptable pipe document, Rule 410, County Ordinance 120108, the Indiana State Department of Health Construction Guidelines for Gravity and Flood-Dose Trench Onsite Systems document, and this document.

Reminders:

  - The lateral holes in a stone and pipe system must be placed at 12 o’clock, 4 o’clock and 8 o’clock if a three hole lateral pipe is used or 4 o’clock and 8 o’clock if a two hole lateral pipe is used.

  - A ninety (90) degree elbow must be installed in the distribution box with a vent in the top.

  - The installer is responsible for abiding by and understanding all the rules and regulations set forth in Rule 410 and County Ordinance 120108.

  - Piping used in a residential on-site sewage system shall meet or exceed the applicable standards in 410 IAC 6-8.3-67.

You can reach the Environmental Specialist daily between the hours of 8:00 a.m. and 4:00 p.m. Monday thru Friday at (574) 223-7730.